COLLEGE OF ENGINEERING & TECHNOLOGY

Department 	: Electronics & Communications Engineering

Lecturer	:  Prof. Mohamed Essam Khedr

Course	:   Communication Networks

Course Code	:    EC 553

### Sheet (6)- Multiple Access Techniques

Top of Form

| 1 |  | If an FDMA network has eight stations, the medium bandwidth has _______ bands. |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | A) | 1 |
|  |  |  | B) | 2 |
|  |  |  | C) | 8 |
|  |  |  | D) | 16 |
| 2 |  | In the _______ random-access method there is no collision. |  |  |
|  |  |  | A) | CSMA/CD |
|  |  |  | B) | CSMA/CA |
|  |  |  | C) | ALOHA |
|  |  |  | D) | Token-passing |
| 3 |  | In the 1-persistent approach, when a station finds an idle line, it _______. |  |  |
|  |  |  | A) | Sends immediately |
|  |  |  | B) | Waits 0.1 s before sending |
|  |  |  | C) | Waits 1 s before sending |
|  |  |  | D) | Waits a time equal to 1 - p before sending |
| 4 |  | _______ requires one primary station and one or more secondary stations. |  |  |
|  |  |  | A) | Token ring |
|  |  |  | B) | Reservation |
|  |  |  | C) | Polling |
|  |  |  | D) | CSMA |
| 5 |  | In the p-persistent approach, when a station finds an idle line, it _______. |  |  |
|  |  |  | A) | Sends immediately |
|  |  |  | B) | Waits 1 s before sending |
|  |  |  | C) | Sends with probability 1 - p |
|  |  |  | D) | Sends with probability p |
| 6 |  | A network using the CSMA random-access method with p equal to 0.25 will send _______ percent of the time after accessing an idle line. |  |  |
|  |  |  | A) | 50 |
|  |  |  | B) | 75 |
|  |  |  | C) | 100 |
|  |  |  | D) | 25 |
| 7 |  | The 1-persistent approach can be considered a special case of the p-persistent approach with p equal to _______. |  |  |
|  |  |  | A) | 1.0 |
|  |  |  | B) | 2.0 |
|  |  |  | C) | 0.1 |
|  |  |  | D) | 0.5 |
| 8 |  | _______ is a random-access protocol. |  |  |
|  |  |  | A) | FDMA |
|  |  |  | B) | CDMA |
|  |  |  | C) | MA |
|  |  |  | D) | Polling |
| 9 |  | In the reservation access method, if there are 10 stations on a network, then there are _______ reservation minislots in the reservation frame. |  |  |
|  |  |  | A) | 10 |
|  |  |  | B) | 11 |
|  |  |  | C) | 5 |
|  |  |  | D) | 9 |
| 10 |  | A Walsh table for 16 stations has a chip sequence of _______ chips. |  |  |
|  |  |  | A) | 16 |
|  |  |  | B) | 32 |
|  |  |  | C) | 4 |
|  |  |  | D) | 8 |
| 11 |  | _______ is a controlled-access protocol. |  |  |
|  |  |  | A) | FDMA |
|  |  |  | B) | TDMA |
|  |  |  | C) | CSMA |
|  |  |  | D) | Reservation |
| 12 |  | _______ is (are) a channelization protocol. |  |  |
|  |  |  | A) | FDMA |
|  |  |  | B) | TDMA |
|  |  |  | C) | CDMA |
|  |  |  | D) | All the above |
| 13 |  | _______ is the access protocol used by traditional Ethernet. |  |  |
|  |  |  | A) | Token ring |
|  |  |  | B) | CSMA |
|  |  |  | C) | CSMA/CD |
|  |  |  | D) | CSMA/CA |
| 14 |  | The most primitive random access method is _______. |  |  |
|  |  |  | A) | Channelization |
|  |  |  | B) | ALOHA |
|  |  |  | C) | CSMA |
|  |  |  | D) | Token passing |
| 15 |  | When a collision is detected in a network using CSMA/CD, _______. |  |  |
|  |  |  | A) | The frame is immediately resent |
|  |  |  | B) | The backoff value is decremented by 1 |
|  |  |  | C) | A jam signal is sent by the station |
|  |  |  | D) | The backoff value is set to 0 |
| 16 |  | In the _______ random-access method, stations do not sense the medium. |  |  |
|  |  |  | A) | CSMA/CA |
|  |  |  | B) | ALOHA |
|  |  |  | C) | CSMA/CD |
|  |  |  | D) | Ethernet |
| 17 |  | When a primary device asks a secondary device if it has data to send, this is called _______. |  |  |
|  |  |  | A) | Backing off |
|  |  |  | B) | Polling |
|  |  |  | C) | Selecting |
|  |  |  | D) | Reserving |
| 18 |  | If a TDMA network has eight stations, the medium bandwidth has _______ bands. |  |  |
|  |  |  | A) | 1 |
|  |  |  | B) | 2 |
|  |  |  | C) | 8 |
|  |  |  | D) | 16 |
| 19 |  | If a CDMA network has eight stations, the medium bandwidth has _______ bands. |  |  |
|  |  |  | A) | 1 |
|  |  |  | B) | 2 |
|  |  |  | C) | 8 |
|  |  |  | D) | 16 |

20- What are the differences between 1-Persistent, Non-Persistent, and P-Persistent CSMA.

21-Host A and B are connected with a Ethernet LAN. CSMA/CD is used in their communication. Suppose the end to end distance between A and B is 500 meters. The Channel speed is 10Mbps and propagation speed is 200m/μsec. Suppose at time 0, A has a 512 bits packet to send to B. and after 2 μsec, B also has a 521b data packet send to A. Will transmission of A and B collide? Describe your reasons. If collision happens, when will A detect it?

Bottom of Form